Workshop in Autonomous Systems Simulation (20973)

Design Review (DR) Document

Autonomous Intelligent Service Robot in a Dynamic Environment

| Platform: | MuJoCo |
| --- | --- |
| Student: | Chani Lerer |
| Submission: | August 31st, 2026 |

# Chapter 1: Introduction & Project Overview

This project involves the development and simulation of an autonomous wheeled service robot operating within a dynamic office environment. The primary objective is to enable the robot to navigate from a designated starting point to a target location efficiently while ensuring high levels of safety. The system must handle real-world complexities, such as stationary furniture and moving obstacles, using reactive and planned navigation strategies.

# Chapter 2: System Requirements

## 2.1 Project Goals

The main goal is to implement a robust autonomous system that maintains operational stability while interacting with a non-static environment. The robot will focus on precision movement and intelligent obstacle avoidance.

## 2.2 Functional Requirements

- Target Acquisition: The robot shall receive specific goal coordinates and plan a path accordingly.

- Sensing: The system shall use Rangefinder sensors to perceive the environment in a 360-degree or multi-directional field.

- Collision Avoidance: The robot must detect and avoid static objects (walls/desks) and dynamic objects (moving entities) without physical contact.

- Real-time Adaptation: The navigation logic must update at a high frequency to respond to sudden changes in the environment.

## 2.3 Non-Functional Requirements

- Performance: The robot should reach its destination within a defined time limit based on path efficiency.

- Code Quality: The software architecture must strictly adhere to S.O.L.I.D. principles to ensure modularity and maintainability.

- Simulation Stability: The simulation must run smoothly on standard hardware without physics-engine instabilities (explosions or jitters).

## 2.4 Environmental Simulation

The simulation, built in MuJoCo, will consist of an office-like arena:

- Static Elements: Four fixed rectangular blocks representing desks.

- Dynamic Elements: Two spherical or cylindrical objects moving in predetermined or random patterns to challenge the robot's reactive logic.

# Chapter 3: System Architecture

The software architecture is designed using a modular approach, strictly adhering to S.O.L.I.D. principles. By decoupling the sensing, logic, and actuation layers, the system remains maintainable and extensible.

## 3.1 Class Structure (Component Breakdown)

To ensure Single Responsibility, the project is divided into the following primary classes:

| Class | Responsibility | Key Functions |
| --- | --- | --- |
| SimulationManager | MuJoCo Interface | Loading MJCF (XML) model, stepping the simulation, managing the render loop |
| SensorSuite | Data Acquisition | Reading raw Rangefinder data and preprocessing it (detecting closest obstacle per quadrant) |
| NavigationPlanner | High-Level Decision Making | Calculating the desired path to target coordinates; deciding when obstacle avoidance is required |
| MotionController | Actuation | Translating navigation decisions into velocity commands for the robot's motors (differential drive) |

## 3.2 System Flow

The system operates in a closed-loop cycle:

- Sense: SensorSuite gathers environmental data.

- Think: NavigationPlanner evaluates the data against the target goal.

- Act: MotionController updates the motor velocities in the MuJoCo simulation.

Architectural Diagram SimulationManager ──► SensorSuite ──► NavigationPlanner ──► MotionController ▲ │ └────────────────── Motor Actuator Commands ────────────────────────┘ Note: The full architectural diagram is included in the Design Review presentation slides.

# Chapter 4: Design Alternatives & Trade-offs

To fulfill the navigation requirements in a dynamic environment, two distinct architectural approaches were evaluated before selecting the final design. The goal was to find a balance between path efficiency and the ability to handle dynamic obstacles in real-time.

## 4.1 Alternative A: Classical Path Planning (Global Approach)

- Concept: Uses algorithms like A* or Dijkstra to calculate an optimal, complete path from the robot's current position to the target based on a static map.

- Pros: Guarantees the shortest possible path and provides predictable, consistent movement.

- Cons: Ineffective against dynamic obstacles. If a moving object blocks the path, the robot must stop and re-calculate the entire route, causing significant latency and potential collisions.

## 4.2 Alternative B: Reactive Navigation (Local Approach)

- Concept: Based on Artificial Potential Fields, where the goal exerts an 'attractive force' and obstacles exert a 'repulsive force' on the robot in real-time.

- Pros: Extremely responsive to sudden changes; requires minimal memory and computation; ideal for avoiding moving objects.

- Cons: Highly susceptible to 'Local Minima' — the robot may get trapped in U-shaped obstacles or corners because it does not 'know' the overall layout of the room.

## 4.3 The Chosen Strategy: Hybrid Navigation System

Decision & Justification: The chosen approach is a Hybrid system. It uses a high-level Global Planner to maintain the general direction toward the target and avoid dead-ends, coupled with a Reactive Avoidance Layer (via the MotionController) to handle real-time maneuvers around dynamic and static obstacles detected by the SensorSuite.

| Criterion | Alternative A (Global) | Alternative B (Reactive) |
| --- | --- | --- |
| Handles dynamic obstacles | ✗ No — requires full replanning | ✓ Yes — responds in real-time |
| Shortest path guarantee | ✓ Yes | ✗ No — may take longer routes |
| Local minima risk | Low | High — can get stuck in corners |
| Computation cost | High on replanning | Low |
| Chosen? | Partially (global layer) | Partially (reactive layer) |

Trade-off Summary: This design prioritizes safety and flexibility without sacrificing global progress. While it increases software complexity, it ensures the robot can navigate a cluttered office space while remaining responsive to sudden environmental changes.

# Chapter 5: System Dynamics & Sequence

This chapter describes how the different components of the system interact during a single simulation step. To maintain a clean architecture, the data flows in a unidirectional loop.

## 5.1 Data Flow Sequence

- Environment State: The SimulationManager retrieves the current world state from MuJoCo.

- Perception: Raw sensor data is passed to the SensorSuite, which calculates the distance to the nearest obstacles in various directions.

- Planning: The NavigationPlanner receives the processed sensor data and the target coordinates. It decides on the steering direction and speed.

- Actuation: The MotionController receives the desired movement and calculates the specific torque/velocity for the left and right wheels.

- Execution: The SimulationManager applies these values back into the MuJoCo engine.

## 5.2 UML Sequence Diagram — Single Simulation Step

The following sequence diagram illustrates the message flow between all system components during a single control loop iteration (100Hz):

Sequence: Single Control Loop Iteration MuJoCo SimulationManager SensorSuite NavigationPlanner MotionController │ │ │ │ │ │── world_state ────►│ │ │ │ │ │── raw_ranges ───►│ │ │ │ │ │── obstacle_map ───►│ │ │ │ │ │── trajectory ────►│ │◄─ apply_commands ──│◄──────────────────────────────────────────── velocity ───│ │ │ │ │ │ │ [loop repeats at 100Hz — 10ms reaction time]

## 5.3 Control Loop Frequency

The control loop will operate at a frequency of 100Hz, ensuring that the robot can react to dynamic obstacles within 10 milliseconds of detection.

# Chapter 6: Key Performance Indicators (KPIs)

To evaluate the success and efficiency of the autonomous system, the following metrics will be measured during simulation trials:

| KPI | Target Value | Description |
| --- | --- | --- |
| Collision Rate | 0% | Any physical contact between the robot and an obstacle (static or dynamic) is considered a failure. |
| Path Efficiency | Ratio ≤ 1.5 | Actual distance traveled ÷ straight-line distance. A ratio of 1.0 is perfect; ≤ 1.5 is acceptable. |
| Time to Goal | ≤ 60 sec (baseline) | Average time to reach the target location across 10 simulation runs with varying obstacle positions. |
| Robustness | ≥ 4/5 recovery attempts | The system's ability to recover and reach the goal after encountering a dead-end or surrounding obstacles. |

# Chapter 7: Risk Assessment & Mitigation

Every engineering project has risks. Identifying them early shows that you are prepared:

| Risk | Impact | Mitigation Strategy |
| --- | --- | --- |
| Local Minima — robot gets stuck in a corner. | High | Implementing a 'recovery behavior' (e.g., backing up or wall-following). |
| Sensor Noise — inaccurate obstacle detection. | Medium | Implementing a simple low-pass filter in the SensorSuite class. |
| Physics Instability — simulation 'explodes' or jitters. | High | Using a smaller time step (dt) in MuJoCo and ensuring proper mass properties. |

# Chapter 8: Project Timeline (Milestones)

Since I am working individually, the project is divided into clear bi-weekly milestones:

| Period | Milestone | Tasks |
| --- | --- | --- |
| Weeks 1–2 | Environment Setup | MuJoCo installation & configuration; MJCF (XML) modeling of the robot and office space. |
| Weeks 3–4 | Sensor & Controller | Implementation of the SensorSuite and basic MotionController (differential drive). |
| Weeks 5–6 | Navigation Planner | Development of the NavigationPlanner (Hybrid approach: global + reactive layers). |
| Weeks 7–8 | Testing & Final Report | Testing, fine-tuning of KPIs across all scenarios, documentation and final report preparation. |

Individual Project  |  Final Submission Deadline: August 31st, 2026

# Chapter 9: Simulation Scenarios

This chapter describes the real-world situations being simulated to test the autonomous robot system. Six scenarios are defined, covering the full range from basic functionality to stress testing.

| ID | Scenario | Description | Type |
| --- | --- | --- | --- |
| S1 | Baseline Navigation | Clear path with no dynamic obstacles. Robot navigates from start to goal. Tests pure path planning and basic motor control. | Functional |
| S2 | Static Obstacle Course | Four desk obstacles form a corridor layout. Tests the global planner's ability to route around fixed furniture. | Pathfinding |
| S3 | Dynamic Avoidance | Two moving objects cross the robot's planned path in random patterns. Tests the reactive avoidance layer exclusively. | Reactive |
| S4 | Full Scenario | Both static desks and two moving obstacles active simultaneously. Full integration test of the hybrid navigation system. | Integration |
| S5 | Dead-End Recovery | Robot is guided toward a U-shaped corner formed by desk obstacles. Tests the recovery behavior (back-up + wall-follow). | Robustness |
| S6 | Stress Test (10 Runs) | Ten repeated runs with randomized start/goal/obstacle positions. Measures KPI consistency and system robustness across diverse conditions. | KPI Eval |

## 9.1 Scenario Details

Scenario S1 — Baseline Navigation

- Environment: Empty office arena, no obstacles.

- Start: (0, 0)  →  Goal: (4, 4) (arbitrary coordinates).

- Success Criteria: Robot reaches goal within 60 seconds with 0 collisions.

- Purpose: Validates basic path planning and motor control before introducing complexity.

Scenario S3 — Dynamic Avoidance

- Environment: Empty arena with two moving cylindrical objects.

- Object motion: Predetermined crossing paths and random wandering patterns.

- Success Criteria: 0 collisions; robot reaches goal despite interference.

- Purpose: Isolates and validates the reactive avoidance layer.

Scenario S5 — Dead-End Recovery

- Environment: Three desks arranged in a U-shape facing the robot's path.

- Expected behavior: Robot detects the dead-end, triggers recovery behavior, backs up, and reroutes.

- Success Criteria: Successful recovery and goal completion in ≥ 4 out of 5 attempts.

- Purpose: Tests the most challenging edge case for the local minima mitigation strategy.

# Chapter 10: Evaluation Criteria

This chapter defines how the system's success will be measured. A project run is considered SUCCESSFUL when all required conditions below are met.

| KPI | Success Threshold | Measured In | Level |
| --- | --- | --- | --- |
| Collision Rate | 0 collisions in ≥ 8/10 runs | All 6 scenarios | Critical |
| Path Efficiency | Average ratio ≤ 1.5 | Scenarios S1–S4 | Required |
| Time to Goal | ≤ 60 sec for S1 (baseline) | Scenario S1 | Required |
| Robustness | Recovery in ≥ 4/5 attempts | Scenario S5 | Required |
| Simulation Stability | 0 crashes or physics failures | Full test suite | Required |

## 10.1 Measurement Methodology

- All KPIs will be measured programmatically via Python logging during simulation runs.

- Collision detection: MuJoCo contact array will be monitored; any non-zero contact force between the robot and an obstacle counts as a collision.

- Path efficiency: Robot position will be logged at each timestep; total path length computed as the sum of Euclidean distances between consecutive positions.

- Time to goal: Simulation timestep counter from spawn to goal-radius arrival (goal radius = 0.3m).

- Each scenario will be run a minimum of 5 times; S6 (stress test) will be run 10 times.

# Chapter 11: Preliminary Prototype Status

This chapter describes the current implementation status at the time of the Design Review.

| Component | Status | Notes |
| --- | --- | --- |
| MuJoCo environment (MJCF) | ✅ Complete | Office arena with 4 desk obstacles and 2 dynamic objects modeled. |
| SimulationManager | ✅ Complete | Simulation loop, rendering, and MuJoCo interface functional. |
| SensorSuite | 🔄 In Progress | Rangefinder reading implemented; quadrant preprocessing pending. |
| MotionController | 🔄 In Progress | Basic differential drive implemented; avoidance tuning pending. |
| NavigationPlanner | ⏳ Planned | Hybrid planner development starts Weeks 5–6. |
| KPI Logging | ⏳ Planned | Will be added during testing phase (Weeks 7–8). |